Verification Plan

• 4-bit ALU performing two-input arithmetic and logic operations

• Reset Signal (Asynchronous active low)

• Result loaded from "A OP B" on a positive clock edge, operation selected by "Opcode" • Opcode encoding: 00= Addition, 01 Subtraction, 10 = AND, 11 = XOR

• Result computed as A + B when Opcode = 00 (Addition)

• Result computed as A - B when Opcode = 01 (Subtraction)

• Result computed as A & B when Opcode = 10 (bitwise AND)

• Result computed as A ^ B when Opcode = 11 (bitwise XOR)

• "Carry/Overflow" flag active high whenever Addition result exceeds 15 (carry out)

"Carry/Overflow" flag active high whenever Subtraction result is negative (borrow)

Signals

1] RST	→ Asynchronous active low "All Output Signals Must be Zero"

2] 4 bit inputs A , B	→ Valid or Invalid (cover full range 0000 to 1111)

3] Opcode[1:0]	→ - @ Posedge CLK  "Selects the ALU Operation"

Set to 00 → Result = A + B  (Addition)

Set to 01 → Result = A - B  (Subtraction)

Set to 10 → Result = A & B  (bitwise AND)

Set to 11 → Result = A ^ B  (bitwise XOR)

4] Result	→ - @ Posedge CLK  "Updated only when RST == 1"

Checker: Result == expected value of the selected operation on the previous A , B

5] Carry/Overflow (Addition)	→ Checker: if Opcode == 00 and (A + B) > 15 → Carry/Overflow == 1 else == 0

6] Carry/Overflow (Subtraction)	→ Checker: if Opcode == 01 and A < B → Carry/Overflow == 1 else == 0

7] Carry/Overflow (AND / XOR)	→ Checker: if Opcode == 10 or Opcode == 11 → Carry/Overflow == 0 always

8] Result (AND)	→ Checker: if Opcode == 10 → Result == (A & B) bitwise, else check invalid

9] Result (XOR)	→ Checker: if Opcode == 11 → Result == (A ^ B) bitwise, else check invalid

10] Opcode	→ Checker: undefined / reserved opcode values are invalid regardless of A , B

11] Carry/Overflow	→ Checker: if Result == 15 in Addition → Check Carry == 1 else == 0

12] Carry/Overflow	→ Checker: if Result == 0 in Subtraction → Check Borrow == 1 else == 0

→ Pulse or All the time

Created by: Fady Samy Nabil	Verification Plan

Functional Coverage

We need to cover all output signals:

Result	→ cover all values (0 to 15)

→ Toggling only

Result\Add	→ Result overflow / carry generated (A + B > 15)

Result\Sub	→ Result underflow / borrow generated (A < B)

Result\AND	→ cover all-zero result and all-one operands

Result\XOR	→ cover all-zero result and all-one result

Carry/Overflow	→ 1 iff (Opcode == Add and A + B > 15)

Carry/Overflow	→ 1 iff (Opcode == Sub and A < B)

Carry/Overflow	→ 0 always when Opcode == AND or XOR

Opcode	→ cover all 4 operations (Add , Sub , AND , XOR)

A , B	→ cover corner values 0000 and 1111 for both operands

Opcode\A\B	→ cross coverage of operation with operand corner cases

Created by: Fady Samy Nabil	Verification Plan

Sequences & Constrained Randomization

1] Reset Sequence	→ with RST == 0

2] Addition Sequence	→ with Opcode == 00

3] Subtraction Sequence	→ with Opcode == 01

4] AND Sequence	→ with Opcode == 10

5] XOR Sequence	→ with Opcode == 11

6] Corner Sequence	→ with A , B == 0000 or 1111 (randomized per operation)

Test Scenarios

Scenario 1 : Addition Sequence for random number transaction range [5:10]

-- Goal : testing addition functionality and carry generation (overflow from 15)

Scenario 2 : Subtraction Sequence for random number transaction range [5:10]

-- Goal : testing subtraction functionality and borrow generation (underflow below 0)

Scenario 3 : AND Sequence for random number transaction range [5:10]

-- Goal : testing bitwise AND functionality, covering all-zero result

Scenario 4 : XOR Sequence for random number transaction range [5:10]

-- Goal : testing bitwise XOR functionality, covering all-one and all-zero result

Scenario 5 : Addition → Subtraction → AND → XOR → Addition … etc.

-- Goal : mixing the order of operations across back-to-back transactions

Scenario 6 : Opcode Sequence → random for at least 10 times

-- Goal : covering all 4 operations with fully randomized operands

Created by: Fady Samy Nabil	Verification Plan