Verification Plan

CXS Bridge Encryption and Data Management System

DUT: top (system level)

• CXS Bridge encrypting payloads arriving over an AMBA CXS link, storing them with parity in the Central Data Memory (CDM), and reporting errors back over the same link

• Two clock domains: CLK_CXS (link side) and CLK_SYS (processing side), joined by an asynchronous FIFO

• Reset Signal (Asynchronous active low), one per domain: rst_n_cxs and rst_n_sys

• Traffic is carried in Transaction Layer Packets (TLPs) of 4 words x 32 bits: 1 header word followed by 3 payload words

• A 256-bit FLIT carries up to two TLPs; cxs_rx_cntl[13:0] marks which 32-bit words are valid

• Header format: BASE_ADDR [31:16], DEV_COUNT [15:8], ENC_MODE [7:4], ADDR_MODE [3], PARITY_MODE [2], SYS_STATE [1:0]

• SYS_STATE selects the phase: 01 = Configuration, 10 = Link-Up, 11 = Transfer

• Configuration TLP writes the global register at CDM 0x0000 and DEV_COUNT device entries from 0x0001 upwards

• Transfer payload word: ADDR [31:16], DATA [15:0]

• Encryption expands every input bit to two output bits, selected by ENC_MODE; a balanced input word overrides the mode

• Parity generates one bit appended above the encrypted word; PARITY_MODE is a single bit, 0 = Even and 1 = Odd

• BASE_ADDR is not hardcoded: software supplies it in the BASE_ADDR field [31:16] of the Configuration TLP header during the Configuration phase, and the ATU latches it and adds it to every payload address during the Transfer phase (Range mode)

• Flow control is credit based in both directions: cxs_rx_crdgnt is granted while the FLIT FIFO has room, and cxs_tx_crdgnt must be granted before a response FLIT is transmitted

Signals

1] rst_n_cxs , rst_n_sys	→ Asynchronous active low "All Output Signals Must be Zero"

Checker: cxs_rx_crdgnt, cxs_tx_valid, cxs_tx_data, cxs_tx_cntl and the async FIFO write enable are all 0 while reset is low; the CDM configuration region is cleared

2] cxs_rx_valid	→ - @ Posedge CLK_CXS "A valid FLIT is present on the link"

Checker: a FLIT is written into the FLIT FIFO only when cxs_rx_valid == 1 and flit_fifo_full == 0, otherwise no write occurs

3] cxs_rx_data [255:0]	→ Checker: the internal 270-bit FLIT == {cxs_rx_cntl, cxs_rx_data}

4] cxs_rx_cntl [13:0]	→ START[j] = bit 0+j , START_PTR[j] = bit 2+j , END[j] = bit 4+j , END_ERROR[j] = bit 6+j , END_PTR[j] = bits 8+3j +: 3

Checker: only the 32-bit words from START_PTR*4 to END_PTR are forwarded to the async FIFO; every other word of the FLIT is dropped

5] cxs_rx_crdgnt	→ Checker: 1 while the receive credit counter is non-zero, 0 once the FLIT FIFO is full

Checker: the credit counter decrements on a FIFO write, increments on a FIFO read, and is unchanged when both happen in the same cycle

6] async_fifo_w_en , async_fifo_w_data	→ Checker: one write per valid 32-bit word, in ascending word order, and never while async_fifo_full == 1

7] direct_cdm_write	→ Checker: 1 for configuration words (encryption path bypassed), 0 for payload words (encryption + parity path)

8] CDM w_en , w_addr , wdata	→ Checker (Configuration): wdata == the received word verbatim, header at 0x0000 and device entries at 0x0001 upwards

Checker (Transfer): wdata == {parity bit, encrypted DATA} written at BASE_ADDR + ADDR

9] dev_count , addr_mode , enc_mode , parity_mode	→ Checker: each equals the matching field of CDM 0x0000 once the configuration header has been written

10] error_valid , error [1:0]	→ 00 = invalid header packet , 01 = invalid ENC_MODE , 10 = invalid state transition

Checker: error_valid is a single CLK_SYS cycle pulse and error holds the code until the next error

11] cxs_tx_valid , cxs_tx_data , cxs_tx_cntl	→ Checker: asserted only when transmit credit is available

Checker: cxs_tx_data[15:8] == 0xE0 response opcode and cxs_tx_data[1:0] == the error code; cxs_tx_cntl has START and END set for the single-word response

12] cxs_tx_crdgnt	→ Checker: each grant allows exactly one transmitted FLIT

Checker: with no credit the encoded FLIT is held and flit_ready is low; the FLIT is sent on the first cycle after a credit is granted

Functional Coverage

Target list for a later phase. The current round is directed only, no coverage is collected.

SYS_STATE	→ cover all four packet types (Idle, Config, Link-Up, Transfer)

error	→ cover all three error codes

ENC_MODE	→ cover 0000 Even, 0001 Odd, and at least one unsupported value

PARITY_MODE	→ cover both values, 0 = Even and 1 = Odd

FLIT occupancy	→ one TLP per FLIT and two TLPs per FLIT

flit_fifo_full	→ FLIT FIFO reaches full and back-pressures the link

async_fifo_full	→ async FIFO reaches full and stalls the packet formatter

Transmit credit	→ credit available and credit exhausted

DEV_COUNT	→ a configuration spanning more than one Configuration TLP

SYS_STATE \ ENC_MODE	→ cross coverage of packet type with encryption mode

Packet slot \ END_PTR	→ cross coverage of FLIT slot with the end pointer value

Sequences

Directed sequences with fixed values. Randomization and constrained sequences are deferred to a later phase.

1] Reset Sequence	→ both resets asserted together, then released

2] Configuration Sequence	→ header with DEV_COUNT = 2 followed by the device entries

3] Link-Up Sequence	→ Link-Up header followed by 3 payload words

4] Transfer Sequence	→ Transfer header followed by 3 payload words

5] Two-TLP FLIT Sequence	→ two Transfer TLPs packed into words 0-3 and 4-7 of one FLIT

6] Error Injection Sequence	→ configuration header carrying an unsupported ENC_MODE

7] Credit Sequence	→ cxs_tx_crdgnt held low, then granted

Test Scenarios

Scenario 1 : CFG_LINK_XFER - Configuration, Link-Up and Transfer, one TLP per FLIT

-- Goal : the full path; configuration registers programmed, then a payload stored encrypted with parity at the translated CDM address

Scenario 2 : TWO_TLP_ONE_FLIT - two Transfer TLPs inside a single FLIT

-- Goal : packet formatter control decode when a FLIT carries two packets

Scenario 3 : ERR_TX_FLIT - configuration header with an unsupported ENC_MODE

-- Goal : error detection, CLK_SYS to CLK_CXS crossing, response FLIT generation and transmission

Scenario 4 : TX_CREDIT_STALL - the same error with no transmit credit

-- Goal : the response FLIT is held in the TX interface until a credit is granted, then sent

Deferred scenarios : FLIT FIFO and async FIFO back-pressure, a configuration spanning several Configuration TLPs, and Region address mode once the ATU implements it.

Open Items (Specification vs RTL)

1] Header field layout	→ PARITY_MODE was reduced from 2 bits to 1 bit, freeing bit [3] for ADDR_MODE, so BASE_ADDR keeps the full 16 bits of [31:16] with no overlap

BASE_ADDR [31:16] , DEV_COUNT [15:8] , ENC_MODE [7:4] , ADDR_MODE [3] , PARITY_MODE [2] , SYS_STATE [1:0]

2] ATU Region mode	→ ADDR_MODE = 1 is not implemented; the ATU only supports Range mode

3] Async FIFO width	→ fifo_mem was instantiated without parameter overrides, truncating the 32-bit word to 8 bits; fixed. The unit test runs at the default width of 8 and did not expose it